DEPA Workshop – Assistant Handbook

This handbook provides step-by-step guidance for assistants managing the DEPA Workshop. The first three days cover robotics, and the final two days focus on IoT telemetry using ESP32 boards, sensors, and cloud data integration.

## 1. Pre-Workshop Setup

1. Run database setup:

python3 database_setup.py

Creates 10 team databases (workshop_team01–workshop_team10) with TimeSeries collections.

2. Verify MongoDB and MQTT credentials in multi_team_mqtt_mongo.js

3. Start the MQTT–Mongo Bridge:

node multi_team_mqtt_mongo.js

Keep this process running during the entire IoT session. Logs confirm message flow.

## 2. ESP32 Setup and Libraries

Ensure each student has the Arduino IDE configured for ESP32 boards. Instruct them to install the following libraries via Arduino Library Manager:

• ArduinoJSON
• PubSubClient
• WiFi
• DHT sensor library
• Adafruit ADXL345 Unified
• Adafruit Sensor
• Wire

## 3. During Workshop

Day 4: Assist teams in wiring sensors and publishing telemetry data to their assigned MQTT topic (teamXX). Each team will use the provided ESP32_MQTT_Mongo.ino code template combining DHT and ADXL345 readings. Ensure they can compile and upload without missing libraries.

Day 5: Help teams export and visualize data using data_export.py and CSV/JSON outputs. Verify they can access MongoDB and understand timestamped data.

## 4. Troubleshooting

• JSON parse error → malformed message; check for proper braces and quotes in JSON payloads.

• No data in MongoDB → confirm bridge running, correct team topic used, and MQTT dashboard shows activity.

• MongoDB connection failed → check internet, credentials, or cluster access permissions.

• ESP32 upload error → check COM port, USB cable, or CP210x drivers.

• Sensor readings all zero → verify correct wiring (SDA/SCL pins for ADXL345, data pin for DHT).

Detailed instructions for ESP related code can be found from “DEPA-INSTRUCT” file

## 5. Maintenance

• Update Node dependencies: npm install

• Recreate databases: python3 database_setup.py

• Backup data: mongodump --uri='<mongodb_uri>' --out=./backup_$(date +%F)

## 6. Key Concepts for Students

• MQTT – publish/subscribe model for telemetry communication

• JSON – structured data used for publishing sensor readings

• TimeSeries Database – efficient for timestamped telemetry data

• Data Pipeline – ESP32 → MQTT Broker → Node.js Bridge → MongoDB → Export/Visualization

• Sensor Integration – students collect temperature (DHT) and acceleration (ADXL345) data, serialized with ArduinoJSON and published every ~0.5s.

## 7. Checklist

✓ Bridge running and logging data

✓ Students connected to Wi-Fi and MQTT broker

✓ Sensor data visible in MQTT dashboard and MongoDB

✓ Export and visualization verified