DEPA Workshop – Student Quickstart Guide

Welcome to the DEPA Autonomous Robot & IoT Telemetry Workshop! Follow these steps to set up your ESP32, collect sensor data, and send it to the cloud for analysis.

## Step 1: Connect to Wi-Fi

• SSID: WorkshopWiFi
• Password: [provided by instructor]

## Step 2: Install Arduino Libraries

In Arduino IDE, go to Tools → Manage Libraries and install:
• ArduinoJSON
• PubSubClient
• WiFi
• DHT sensor library
• Adafruit ADXL345 Unified
• Adafruit Sensor
• Wire

## Step 3: Configure MQTT

• Server: automaatio.cloud.shiftr.io
• Port: 1883
• Username: automaatio
• Password: [provided]
• Topic: teamXX (replace XX with your team number)

## Step 4: Publish Sensor Data

Use the provided ESP32_MQTT_Mongo.ino example. Your code should publish both DHT and ADXL345 sensor data in JSON format:

Detailed instructions on ESP setup for steps 2-4 can be found in the DEPA-INSTRUCT.docx word document.

## Step 5: Verify Data

Open https://automaatio.cloud.shiftr.io to see live MQTT messages or ask an assistant to confirm your team data in MongoDB.

## Step 6: Export Data

Run: python3 data_export.py
Exports your telemetry to teamXX_sensor_data.csv and .json for analysis.

## Step 7: Visualize Your Results

Open the CSV file in Excel, Google Sheets, or Colab. Plot acceleration or temperature over time.

Remember:
• Keep JSON syntax valid (use quotes around keys)
• Stay on your assigned topic (teamXX)
• Publish new data every ~0.5 seconds
• Test sensors individually before combining them
• Export and visualize your data for your report